| How to make an apparatus for sensing Volatile organic compounds (VOC) – WORKSHEET |  |  |  |
| --- | --- | --- | --- |
| Material and tools: - Multimeter - Breadboard - DC source 5V-9V - Electrical wires - VOC sensor (TGS 2620) - Resistor 1 kΩ - Resistor 47 kΩ - LED |  |  |  |
| Photo and schematics of the aparatus |  |  |  |
| Instructions | Tools and materials |  | Photo instructions |
| - Pin on the breadboard: - LED at + electrode in B21 and – electrode in B22 - Connect the resistor with the 1kΩ resistance with the + electrode and A21 pin - Connect pin A22 and - electrode | - LED diode - Resistance 1kΩ - Electronic wire |  |  |
| - Pin on the breadboard: - 47kΩ resistor from E2 to - line - Connect with wires - lines (directly or how it’s showed on the picture) | - Resistor 47kΩ - Electronic wires |  |  |
| - Pin on the breadboard: - VOC sensor with it’s nodge turned towards the middle of the board; legs A,B,C and D into lines 1,2,3 and 4 - Wire from A1 to the - line - Wire from A3 to the + line - Wire from A4 to the + line | - VOC sensor - Electronic wires |  |  |
| - Connect voltmeter on the breadboard: - Connect multimeter terminal VΩmA with red wire to - line - Connect multimeter terminal COM with black wire to A2 | - Multimeter - Red wire - Black wire |  |  |
| - Connect DC current source in the electrical circuit in the following way: - + line on + electrode - - line on - electrode | - DC current source |  |  |